## Maseeh College of Engineering and Computer Science

Department of Civil and Environmental Engineering

Post Office Box 751	503-725-4282 tel

Portland, Oregon 97207-0751	503-725-5950 fax

CE443(543) Introduction to Geotechnical Earthquake Engineering

To:         CE443(543) Students								Date: January 15, 2026

From:     Arash Khosravifar

Subject: Homework Assignment #2 (Due Jan 23th at 11:59 p.m.)

Problem 1:

In this question, we will investigate the differences in ground motions at a soil site and a rock site located close to each other near Gilroy, California. The Gilroy 1 site is a rock site, and it is located about 2 km west of Gilroy 2, a soil site. Strong ground motion instruments at these two sites recorded the 1979 Coyote Lake Earthquake, the 1984 Morgan Hill Earthquake, and the 1989 Loma Prieta Earthquake.

Locate the "PEER Ground Motion Database" website by the Pacific Earthquake Engineering Research (PEER). From this page, or using a general web search, find the "NGA-West2" where NGA stands for Next Generation Attenuation. At the NGA-West 2 website, find the Databases tab and download the “NGA-West2 Flatfile” compressed file (527 MB). There are 11 files inside the compressed file that report pseudo-spectral accelerations for damping ratios ranging from 0.5% to 30%. Locate the one corresponding to 5% damping (also uploaded on Canvas). This Excel spreadsheet has a wealth of information regarding available strong ground motion records. Locate the Final Products tab and download the “PEER 2013/03 PEER NGA-West2 Database” report (PDF file, 6 MB), where you can find the explanation for all the headings in the flatfile under Appendix D of the report.

(a) For each of the above three earthquakes, summarize the fault name, the earthquake moment magnitude, fault rupture length, and fault mechanism. Plot the combinations of magnitude and fault rupture length on the graph by Wells and Coppersmith (1994) as given in the course notes. How do these earthquakes compare to the median relation in this correlation – are they above or below the mean relation, and are they within the scatter of the data?

(b) For both sites, list the peak horizontal acceleration and peak horizontal velocity recorded during these earthquakes. Note that the "peak" value that you obtain from the flatfile is actually the geometric mean of the peak values from the two orthogonal horizontal directions. Plot the peak horizontal acceleration at Gilroy 2 versus peak horizontal acceleration at Gilroy 1. Make the same plot for peak velocity. How do the motions at the soil site compare to those at the rock site?

(c) Determine the ratio of the pseudo spectral accelerations (5% damping) for the horizontal motions at Gilroy 2 (soil) versus at Gilroy 1 (rock). Determine these ratios for each of the three earthquakes, and plot them versus period. Plot all three earthquakes on one graph. Make the "period" axis to be in logarithmic scale (0.01 to 10 sec), and the spectral ratios in arithmetic scale. How do the spectral accelerations on soil compare to those on rock? At what periods are the differences greatest? Do the differences vary with earthquakes? If you see any differences between ratios for these earthquakes, explain why they are different. Can you relate your observations to the plots from (b)?

(d) Suppose a water tower that acts as a linear-elastic single-degree-of-freedom structure with 5% damping was located at the Gilroy 2 station during the 1984 Morgan Hill earthquake. The tower's superstructure weighs 60 kN, and its natural period is 1.0 second. Using the pseudo spectral accelerations obtained from the NGA flatfile, estimate the tower's peak horizontal base shear force during this earthquake. Estimate the peak horizontal displacement of the superstructure relative to its base under this peak loading.

Hint for Problem 1c:

The pseudo spectral accelerations (PSA) are listed in the NGA flatfile in columns EE to IK. This is explained in Appendix D of the PEER report (except below). The headings of these columns specify the periods. For example, the heading for Column EE is "T0.010S" which means T=0.01 sec. To be able to plot PSA vs T you will have to delete "T" and "S" from the headings. You can either do that manually (for 111 columns) or use the Convert Text to Columns Wizard in Excel. You can find those period values in “Periods.xlsx” uploaded on Canvas. Feel free to come to the office hours if you want to learn how to do this data manipulation in Excel.